CubeSat Command and Data Handling (CDH) Subsystem Documentation

- Introduction

  - The Command and Data Handling (CDH from now on) subsystem is the central “brain” of the spacecraft. It manages spacecraft modes, coordinates subsystems operations, stores and forwards telemetry, and ensures reliable commanding from the ground. The CDH also supervises safing behavior to protect the spacecraft under fault conditions

- CDH Responsibilities

  - Execute the spacecraft’s mode logic (SAFE, IDL, NOMINAL, SCIENCE, LOW POWER)

  - Handle command routing from ground and distribute to subsystems (EPS, ADCS, COMM, pay)

  - Collect and store telemetry from all boards

  - Perform safing actions when fault are detected

  - Support firmware updates over the ground link with triple redundancy

  - Provide time bas for scheduling and synchronization

  - Monitor spacecraft health and provide watchdog recovery

- System Architecture

  - Hardware Platform

    - Microcontroller: STM32U5 family (ARM Cortex-M33)

    - Interfaces:

      - CAN Bus for subsystem communication

      - FRAM/MRAM for telemetry ring buffer

      - Dual-bank flash with TMR copies for firmware images.

    - Watchdog:  independent watchdog (IWDG)

  - Interfaces and Data Flows

    - Ground <-> CDH (via radio): Commands, firmware updates, telemetry downlink.

    - CDH <-> Subsystems (via CAN): Commands and state of health (SOH) telemetry.

    - Internal Storage: Telemetry ring buffer and firmware images.

- Operational Flow

  - Boot Sequence

    - Power-on reset -> initialize clocks, watchdog, SysTick

    - Check boot markers and firmware bank health (triple-redundant vote)

    - Initialize CAN, radio, and storage

    - Perform self-checks with EPS, COMM, ADCS

    - Enter SAFE Mode as default startup state

  - Main Loop Activities

    - Kick Watchdog

    - Process incoming radio frames -> commands

    - Process incoming CAN frames -> subsystem SOH/commands

    - Rout commands (local handling or forwarding)

    - Store telemetry to ring buffer

    - Downlink beacons or telemetry opportunistically

    - Run mode manager and safing supervisor

    - Perform background memory scrub

    - Update heartbeat LED

- Modes and State Machine

  - MODES

    - BOOT: Early startup

    - SAFE: Minimum power, radios beacons at low rate, payload disabled

    - IDLE: Ready state, normal power, limited activity

    - NOMINAL: Full operation of bus + payload

    - SCIENCE: Payload active, higher data flow

    - LOW POWER (PHOENIX): Reduced MCU frequency, minimal comms, watchdog alive

  - Transistions

    - Enter SAFE on faults (low battery, thermal, comms loss)

    - Leave SAFE only by ground command or verified recovery

    - IDLE -> NOMINAL -> SCIENCE based on mission timeline and ground command

- Command and Data Handling

  - Command Pipeline

    - Radio input: framed, CRC-validated commands from ground

    - CAN input: multi-frame subsystem commands

    - Router actions:

      - Local commands (PING, SET_MODE, FW ops, telemetry requests)

      - Forwarded commands (EPS,ADCS,COMM,Pay)

  - Telemetry Pipeline

    - Sources: CDH SOH, subsystem SOH (via CAN), event logs

    - Storage: ring buffer with CRC protection

    - Downlink: opportunistic or on ground request

    - Beacons: periodic SOH frames (10s SAFE, 2-5s NOMINAL)

- Firmware Update Process

  - Dual-bank flash layout (Bank A / Bank B)

  - Each bank stored in three redundant copies

  - Update flow:

    - FW_BEGIN -> erase three copies

    - FW_WRITE_CHUNK -> stream data into all three copies

    - FW_END -> verify CRC majority vote

    - FW_COMMIT -> mark bank active and reboot

  - Background scrub: byte-level majority vote and repair of dissenters.

  - Rollback: If new bank fails verification at boot, revert to previous bank and SAFE

- Safing Supervisor

  - Monitors

    - Battery voltage from EPS

    - Temperature from onboard sensors

    - Comms timeout (no ground link or EPS ping)

    - Error counters (command bursts, CAN faults)

  - Actions

    - Force SAFE mode

    - Command EPS to shed loads

    - Zero ADCS torque

    - Record safing cause for post-mortem

- Scheduling and Timing

  - Watchdog kick ~200 ms

  - SOH telemetry: 1s

  - Beacons: 10s SAFE, 2-5s NOMINAL

  - Firmware scrub slide 30s

  - COMMS timeout: 5s

  - EPS Ping: 10-30s

- The CDH orchestrates spacecraft activity by providing reliable command routing, telemetry storage, and fault protection. Its design emphasizes robustness (triple-redundant firmware, watchdogs, scrubbing), simplicity (single cooperative loop, no RTOS), and safety (default SAFE mode, strict guards on transitions)